To correctly print labels (in either DOS or Windows) using the ZEBRA TLP 2844 thermal printer I will use the following example:

Project:

- We want to print on sticker labels (4.5X7)cm size thermal labels, some customers’ addresses from the “cust.dbf” file.

- The fields we would like to print are the following 6:

- NAME             : 15 c

- LAST                : 15 c

- STREET           :  20 c

- ZIPCODE         :   5 c

- TOWN             :  15 c

- CUSTCODE     :   5 n

We must perform the following actions

We download the form file (for example “label.ejf”) to the printer(either USB or LPT). The form file can be created with a text editor like notepad and it must follow a specific pattern:

OD ; *

I8,D

FK"LABEL” ; We are deleting the form “LABEL” from the printer’s memory 2 times

FK"LABEL” ; This specific model ZEBRA TLP2844 needs that to be done twice.

FS"LABEL" ; We are starting to create the new form

V00,15,L,"v1"  ; NAME**

V01,15,L,"v2"  ; LAST ***

V02,20,L,"v3"  ; STREET

V03,5,L,"v4"    ; ZIPCODE		// We are setting up the variables depending on the data we’ll send

V04,15,L,"v5"  ;TOWN

V05,5,L,"v6"    ; CUSTCODE

Q355,50

q560

S3

D10

ZB

A20,15,0,4,1,1,N,V00

A20,60,0,4,1,1,N,V01

A20,105,0,4,1,1,N,V02

A20,150,0,4,1,1,N,V03		        	   // The printing position of every variable on the label

A20,195,0,4,1,1,N,V04

A20,240,0,4,1,1,N,"CustCode:"

B180,240,0,1,2,1,40,B,V05		   // This will print a barcode of the CUSTCODE

FE ; We end the form

_ _ _ _

*   The printers’ commands can be found inside EPL manual(EPL Manual.pdf.

**  We have to send always exactly the same amount of variables we have inside our print txt file. In our case 6.

*** We can type any comments we want after the “;”

In case we have an LPT connection (Parallel Port) ,  we download the form to the printer through LPT or serial port.

RUN COPY  LABEL.EJF   LPT1  >NUL

The above setups the printer who from now on waits for data in the specific form we send him from the file “label.ejf”

In case we have a USB connection: The following example works also in a server environment(connected through desktop remote control) and TLP2844 printers connected on the local PCs. In that case every printer must have a different name. For example if we have 3 Pcs and each one has 1 printer, we must name them accordingly: On the 1st pc we name the printer”Zebra1”, on the 2nd pc we name the printer”Zebra2” an on the 3rd pc we name the printer as “Zebra3”.In our exampleour printer will be named “LabelPrinter” and the form file “label.ejf”.

At first we check if the printer is available and we select it..

(Look inside  LABELZEB.PRG)

cFILE :=  "LABEL.EJF"

cPrinter  := SelectPrn(LabelPrinter)

if empty(cPrinter)

msgstop( “Printer not found” )

quit

endif

Win_PrintFileRaw (cPrinter, cFILE)  // Downloads  the form file to the selected printer.

************************************

FUNCTION SelectPrn(cPrn)

Local aTempArray := {}

aTempArray := aPrinters()

If len(aTempArray) < 1

Return NIL

Else

For i := 1 to len(aTempArray)

If UPPER(Left(aTempArray[i], len(alltrim(cPrn)))) == UPPER(cPrn)

Return alltrim(aTempArray[i])

EndIf

Next i

EndIf

- Creating The Labels File

ASCII Label File

I can create it in 2 ways:

- Using the MEMOWRIT() function

Look inside LABELZEB.PRG  the  Function  MAKELABFILE(cFormFile)  // With MEMOWRIT()

- Using the  ALTERNATE command

Look inside  LABELZEB.PRG the  Function  MakeAltrnFile(cFormFile) // with ALTERNATE

- Printing The Label File

  - We can print the labels from an ascii file through USB,LPT or SERIAL port using the Function Win_PrintFileRaw(),

  - Or directly through the printer using LPT or SERIAL port by printing 1 by 1 label:

SET CONS OFF

SET PRINTER TO LPT2 // If we are printing to LPT and DBF file is too big(so we have a lot of labels),its  better to print directly to Printer every label one by one...

SET PRINTER ON

Do !eof()

? data

.

.

.

Enddo

SET PRINTER OFF

SET PRINTER TO

SET CONS ON

Look inside LABELZEB.PRG the Function  MakeAltrnFile(cFormFile)  line 127

Attached Files:

- Label_Read.doc  =  This file

- Labelzeb.prg  = the main file

- Main.fmg  = the main form

- Label.ejf   = form file which will be downloading to the printer, for thermal* labels 4,5X7 size

- Cust.dbf   =  Date for the labels

- __labelFile.txt  = a sample of the labels file created

- Reset.txt = a file which contains the commands for resetting the printer

- EPL Manual.pdf = Documentation for the EPL commands used in the printer’s form

* No need for ink ribbons